Table of Contents

Reading Potentiometer	1

Control One Servo with the Potentiometer	1

Control One Servo with a Joystick	3

Control Two Servos with a Joystick Module	5

## Reading Potentiometer

- Potentiometer is connected as follows:  Middle = pin D10, Side 1 =  3.3V power pin from microcontroller, Side 2 = GRD

- Potentiometer middle pin may be connected to any of these pins: D0, D1, D2, D3, D4, D5, D8, D9, D10

const int POT_PIN = D10;

void setup() {

Serial.begin(115200);

analogReadResolution(12);

}

void loop() {

int reading = analogRead(POT_PIN);

Serial.println(reading);

delay(100);

}

- By turning the screw on the potentiometer:

  - Minimum reading: 0

  - Maximum reading: 4095

  - Reading variance is +/-20 when potentiometer screw does not change. This number is important to determine the deadband later.

## Control One Servo with the Potentiometer

- Potentiometer connection same as above

- Servo is connected to pin D7 and has a SEPARATE 5V power supply (in this case, it’s a 9V batter + regulator that steps down to 5V and is capable of continuous 3A)

- MAKE SURE ALL GNDS CONNECTED

#include <ESP32Servo.h>

// pin connections

const int POT_PIN = D10;

const int SERVO_PIN = D7;

Servo myServo;

// these are used for deadband

int oldPotValue;

int servoAngle = 90;

void setup() {

// create serial readings

Serial.begin(115200);

myServo.attach(SERVO_PIN);

// create the intial potentiometer reading integer variable

oldPotValue = analogRead(POT_PIN);

}

void loop() {

int newPotValue = analogRead(POT_PIN);

// FOR DEADBAND

int change = newPotValue - oldPotValue;

// Make a negative change positive

if (change < 0) {

change = -change;

}

// Only update the servo when the potentiometer

// changes by more than 30

if (change > 30) {

servoAngle = map(newPotValue, 0, 4095, 0, 180);

myServo.write(servoAngle);

oldPotValue = newPotValue;

}

// Print

Serial.print("Potentiometer: ");

Serial.print(newPotValue);

Serial.print("    Servo angle: ");

Serial.println(servoAngle);

delay(50);

}

## Control One Servo with a Joystick Module

- Joysticks power is connected to 3V3 on the esp32 (its labelled 5V though)

- Start with using just 1 DOF on the joystick (Vx)

- In this code, the center of the joystick puts the servo at 90deg (center), the left side puts the servo to 0deg (one boundary) and the right side of the joystick puts the servo to 180deg (other boundary).

- A dead zone needs to be included. This can be measured by leaving the joystick in the center position and noting the min and max values that the Joystick reads. In my case, its 1820-1870, and these numbers will be used in the code for deadband:

#include <ESP32Servo.h>

const int JOYSTICK_PIN = D10;

const int SERVO_PIN = D7;

Servo myServo;

void setup() {

// baud rate of serial info sent to the computer

Serial.begin(9600);

myServo.attach(SERVO_PIN);

}

void loop() {

// Read joystick position

int joystickReading = analogRead(JOYSTICK_PIN);

int servoAngle;

// Joystick centre is approximately 1850.

// Ignore small changes near the centre (deadband) so the servo doesnt shake in the middle

if (joystickReading > 1820 && joystickReading < 1870) {

servoAngle = 90;

}

else {

// Use the full joystick range

// Jostick readings are 0-4095 left->right and the servo motor goes 0-180 deg

// map(value, inputMinimum, inputMaximum, outputMinimum, outputMaximum)

servoAngle = map(joystickReading, 0, 4095, 0, 180);

}

// Move the servo

myServo.write(servoAngle);

Serial.print("Joystick: ");

Serial.print(joystickReading);

Serial.print("    Servo angle: ");

Serial.println(servoAngle);

delay(20);

}

- Observation: when joystick in middle, servo in middle. Move joystick all the way to right, servo moves right 90deg. Move all the way to left, servo moves 90deg left. Servo travels a total 180 degs.

## Control Two Servos with a Joystick Module

- In this case, the Vx axis of the joystick controls one servo and the Vy axis of the joystick control the second servo.

- Need to add a second power supply for the other servo (mikes voltage regulator + 9V battery)

- Before adding the Vy pin with the Vx pin for the joystick, test just the Vy to find its center so it doesn't shake in the middle. Aka find its deadband the same way the deadband in the x direction was found

  - For Vy, while the joystick is in the middle, its readings are between 1890-1930

  - For Vx, its 1820-1870

Code:

#include <ESP32Servo.h>

const int JOYSTICK_X_PIN = D10;

const int JOYSTICK_Y_PIN = D9;

const int SERVO1_PIN = D7;

const int SERVO2_PIN = D8;

Servo servo1;

Servo servo2;

void setup() {

// baud rate of serial info sent to the computer

Serial.begin(9600);

servo1.attach(SERVO1_PIN);

servo2.attach(SERVO2_PIN);

}

void loop() {

// Read joystick position

int joystickReading_x = analogRead(JOYSTICK_X_PIN);

int joystickReading_y = analogRead(JOYSTICK_Y_PIN);

int servo1Angle;

int servo2Angle;

// clearly the deadband for Vy direction of the joystick is off from the x direction

if ( joystickReading_x > 1820 && joystickReading_x < 1870 ) {

servo1Angle = 90;

}

else {

//map(value, inputMinimum, inputMaximum, outputMinimum, outputMaximum)

servo1Angle = map(joystickReading_x, 0, 4095, 0, 180);

}

if (joystickReading_y > 1890 && joystickReading_y < 1930) {

servo2Angle = 90;

}

else {

servo2Angle = map(joystickReading_y, 0, 4095, 0, 180);

}

// Move the servos

servo1.write(servo1Angle);

servo2.write(servo2Angle);

Serial.print("Joystick x: ");

Serial.print(joystickReading_x);

Serial.print("    Joystick y: ");

Serial.print(joystickReading_y);

Serial.print("    Servo 1 angle: ");

Serial.print(servo1Angle);

Serial.print("    Servo 2 angle: ");

Serial.println(servo2Angle);

delay(20);

}

## Control Two Servos with Only one Axis on the Joystick Module – Opposing Motion Creation

- The purpose of this is to practise coordinated opposing actuator control

- For the final sleeve, we want:

Wrist correction in direction 1: Servo 1 pulls its tendon, Servo 2 releases its tendon

Wrist correction in direction 2: Servo 2 pulls its tendon Servo 1 releases its tendon

The joystick position represents the desired correction:

- Centre: no wrist correction.

- One direction: correct the wrist one way.

- Opposite direction: correct the wrist the other way.

- Greater joystick displacement =  greater correction.

Tips: Dont hardcode deadband center as the value seems to shift over time

#include <ESP32Servo.h>

// ---------------- PIN CONNECTIONS ----------------

// This is the only joystick axis used to control the servos.

const int JOYSTICK_X_PIN = D10;

// The Y-axis is not used for control

// but it is read and printed for testing.

const int JOYSTICK_Y_PIN = D9;

// Servo signal pins.

const int SERVO1_PIN = D7;

const int SERVO2_PIN = D8;

// ---------------- SERVO SETTINGS ----------------

// Starting position for each servo.

const int SERVO1_NEUTRAL = 90;

const int SERVO2_NEUTRAL = 90;

// Maximum distance each servo can move away from 90°.

// Lowest angle  = 90 - 30 = 60°

// Highest angle = 90 + 30 = 120°

// this works as a "softstop"!! Actuaotr wont go more than 30deg each direction.

const int MAX_OFFSET = 30;

// ---------------- JOYSTICK SETTINGS ----------------

// Small joystick changes near the centre are ignored.

// This prevents sensor noise from making the servos shake.

// let the deadband be 30 counts of the joystick in either direction,

// (total 60 counts) this is more than enough based on past testing

const int JOYSTICK_DEADBAND = 60;

// The actual centre value will be measured automatically

// when the program starts. Create the varaible for that

// Measure it for each test and dont hardcode the deadband,

// as previous tests have shown the deadband shifts over time

int joystickCenterX = 0;

// Create the two servo objects.

Servo servo1;

Servo servo2;

// Read the joystick several times and calculate the average.

// Averaging helps reduce small changes caused by electrical noise.

// This function reads the joystick x pin 10x and returns the avg reading

int readAverageJoystick(int pin) {

const int NUMBER_OF_SAMPLES = 10;

long total = 0;

for (int i = 0; i < NUMBER_OF_SAMPLES; i++) {

total += analogRead(pin);

delayMicroseconds(200);

}

return total / NUMBER_OF_SAMPLES;

}

void setup() {

// Start communication with the Serial Monitor.

Serial.begin(9600);

// ESP32 analog readings will range from 0 to 4095.

analogReadResolution(12);

// Connect each Servo object to its signal pin.

servo1.attach(SERVO1_PIN);

servo2.attach(SERVO2_PIN);

// Begin with both servos at 90°.

servo1.write(SERVO1_NEUTRAL);

servo2.write(SERVO2_NEUTRAL);

delay(500);

// Measure the joystick's centre position.

//Do not touch the joystick while the board is starting.

//The program averages 200 readings to find the centre.

long total = 0;

for (int i = 0; i < 200; i++) {

total += analogRead(JOYSTICK_X_PIN);

delay(5);

}

joystickCenterX = total / 200;

Serial.print("Measured joystick X centre: ");

Serial.println(joystickCenterX);

delay(500);

}

void loop() {

// Read the joystick axes.

int joystickReadingX = readAverageJoystick(JOYSTICK_X_PIN);

int joystickReadingY = readAverageJoystick(JOYSTICK_Y_PIN);

/*

offset tells us how far the servos should move away

from their neutral positions.

offset = 0:

Servo 1 = 90°

Servo 2 = 90°

offset = +30:

Servo 1 = 120°

Servo 2 = 60°

offset = -30:

Servo 1 = 60°

Servo 2 = 120°

*/

// create offset varaible

int offset = 0;

// Joystick moved in the positive X direction.

if (joystickReadingX > joystickCenterX + JOYSTICK_DEADBAND) {

offset = map(

joystickReadingX,

joystickCenterX + JOYSTICK_DEADBAND,

4095,

0,

MAX_OFFSET

);

}

// Joystick moved in the negative X direction.

else if (joystickReadingX < joystickCenterX - JOYSTICK_DEADBAND) {

offset = map(

joystickReadingX,

0,

joystickCenterX - JOYSTICK_DEADBAND,

-MAX_OFFSET,

0

);

}

/*

If the joystick is inside the deadband,

neither condition runs and offset remains zero.

*/

// Ensure that the offset remains within the safe test range.

offset = constrain(offset, -MAX_OFFSET, MAX_OFFSET);

// Give the two servos opposite angle commands.

int servo1Angle = SERVO1_NEUTRAL + offset;

int servo2Angle = SERVO2_NEUTRAL - offset;

// Send the commands to the servos.

servo1.write(servo1Angle);

servo2.write(servo2Angle);

// Print

Serial.print("Joystick X: ");

Serial.print(joystickReadingX);

Serial.print("    Joystick Y: ");

Serial.print(joystickReadingY);

Serial.print("    Offset: ");

Serial.print(offset);

Serial.print("    Servo 1: ");

Serial.print(servo1Angle);

Serial.print("    Servo 2: ");

Serial.println(servo2Angle);

delay(20);

}